MAF Translator and Translator Plus Installation

The MAF Translator and Translator Plus are easy to install.  The MAF Translator is a simple plug-in, while the Translator Plus requires connection of the Spark signals into the vehicle’s electrical harness.  Having the factory service manual is a great help in installing these devices.

Test fit all connections described below before selecting a mounting location for the unit.  A variety of extension harnesses are available to allow mounting the unit almost anywhere.  When mounting the unit under the vehicle’s hood, select a location away from sources of heat and water splash/drainage.

Be sure to route all wires away from spark plug and coil wires to avoid interference.

Connecting the MAF signals:

- Locate the MAF (Mass Air Flow) sensor.  It is in the path of incoming engine air between the air filter and the throttle body.

- Unplug the factor harness connector, bend the lock tab out slightly and gently unplug.

- In the case of a sensor retrofit, replace the factory sensor with the new one.

- Plug the MAF Sensor pigtail of the Translator into the MAF

- Plug the Vehicle harness into the MAF Harness Pigtail of the translator.  In case of certain sensor retrofit cases an adapter harness may be necessary.

CAUTION, the connector from the Translator to the Vehicle harness is NOT KEYED and can be inadvertently reversed.  Reversing the connector will damage the MAF.  Double and triple check the connection.  The Pink wires must line up.  The Yellow wire in the harness must line up with the Green wire from the translator.  The connector pin letters will not match.  BE SURE TO CHECK AND RECHECK THIS CONNECTION.

CAUTION, the MAF signal connector from the translator may be difficult to engage in the vehicle harness connector.  This is due to mis-alignment of the connector pins.  If the connectors will not push together with moderate force inspect/correct the pin centering in the MAF signal output connector.

CAUTION, the vehicle harness connector pins may have corroded or aged.  Intermittent connections will cause unpredictable results.  Please inspect, clean, and/or re-tension the connector pins during the installation process.

Connecting the Spark signals (Translator Plus only):

- Locate the EST signal wire.  On GM vehicles this wire is almost always white and runs from the ECM/PCM to the coil/module.  This wire controls the firing of the ignition coil and thus the spark advance timing.  This wire must be cut and spliced unless a plug-and-play adapter has been purchased.

- If a Plug-and-Play adapter has been purchased

- ,  install it according to it’s instructions.  Otherwise cut the EST wire with enough slack to be able to strip and crimp (or solder) on the Spark pigtail.  The white wire of the pigtail goes to the wire that connects to the ECM/PCM.  The Gray wire goes to the wire that connects to the Coil/Module.  Before crimping or soldering the connections slide a piece of shrink tubing over the end of the pigtail wires.  Crimp or twist+solder the pigtails wires to the cut ends of the vehicle harness and cover with the shrink tubing.  Heat the shrink tubing with a heat gun or a match/lighter (CAREFULLY!) to seal the splices.

- Plug the Spark pigtail from the Translator Plus into the Vehicle Pigtail or the Plug-and-Play adapter.

- Put the Spark signal jumper plug in a safe place.  In case of removal of the Translator Plus from the vehicle it can be plugged into the vehicle pigtail to reconnect the EST signal.